KC868-A16 Modbus RTU Register Map
(ESP32 Slave / VB.NET Master)

Document version: 1.0   |   Generated: 2026-01-11

# 1. Modbus addressing and data conventions

This project uses Modbus RTU. Tables below are written in Modbus reference form (00001 / 10001 / 30001 / 40001). Most PC libraries (including NModbus4) use 0-based addressing. Example: 40001 corresponds to address 0 when calling ReadHoldingRegisters().

## 1.1 Addressing

| Reference | Meaning | NModbus4 address (0-based) |
| --- | --- | --- |
| 00001 | Coil #1 | 0 |
| 10001 | Discrete Input #1 | 0 |
| 30001 | Input Register #1 | 0 |
| 40001 | Holding Register #1 | 0 |

## 1.2 Data encoding

Unless stated otherwise:

U16 = unsigned 16-bit integer (0..65535)

S16 = signed 16-bit integer (-32768..32767)

U32 = 2 registers, LOW word first then HIGH word (little-endian word order)

FLOAT32 = IEEE754 float stored in 2 registers, LOW word first then HIGH word (little-endian word order)

STRING(N) = ASCII string; 2 characters per register (high byte then low byte), padded with 0x00

Note: For FLOAT32/U32, the “low word first” rule means the first register contains bits 15..0 of the value.

# 2. Register map overview

Device role: ESP32 (KC868-A16) acts as Modbus RTU Slave/Server (default Slave ID = 1). VB.NET WinForms acts as Modbus RTU Master (NModbus4).

## 2.1 Coils (0xxxx) — Digital outputs and modes

Writing coils provides real hardware control. OUTPUTS_MASTER_ENABLE overrides output control when set to 0.

| Ref | Offset | R/W | Type | Name | Description |
| --- | --- | --- | --- | --- | --- |
| 00001 | 0 | R/W | Coil | DO1 | MOSFET output channel control (0=OFF, 1=ON). |
| 00002 | 1 | R/W | Coil | DO2 | MOSFET output channel control (0=OFF, 1=ON). |
| 00003 | 2 | R/W | Coil | DO3 | MOSFET output channel control (0=OFF, 1=ON). |
| 00004 | 3 | R/W | Coil | DO4 | MOSFET output channel control (0=OFF, 1=ON). |
| 00005 | 4 | R/W | Coil | DO5 | MOSFET output channel control (0=OFF, 1=ON). |
| 00006 | 5 | R/W | Coil | DO6 | MOSFET output channel control (0=OFF, 1=ON). |
| 00007 | 6 | R/W | Coil | DO7 | MOSFET output channel control (0=OFF, 1=ON). |
| 00008 | 7 | R/W | Coil | DO8 | MOSFET output channel control (0=OFF, 1=ON). |
| 00009 | 8 | R/W | Coil | DO9 | MOSFET output channel control (0=OFF, 1=ON). |
| 00010 | 9 | R/W | Coil | DO10 | MOSFET output channel control (0=OFF, 1=ON). |
| 00011 | 10 | R/W | Coil | DO11 | MOSFET output channel control (0=OFF, 1=ON). |
| 00012 | 11 | R/W | Coil | DO12 | MOSFET output channel control (0=OFF, 1=ON). |
| 00013 | 12 | R/W | Coil | DO13 | MOSFET output channel control (0=OFF, 1=ON). |
| 00014 | 13 | R/W | Coil | DO14 | MOSFET output channel control (0=OFF, 1=ON). |
| 00015 | 14 | R/W | Coil | DO15 | MOSFET output channel control (0=OFF, 1=ON). |
| 00016 | 15 | R/W | Coil | DO16 | MOSFET output channel control (0=OFF, 1=ON). |
| 00019 | 18 | R/W | Coil | OUTPUTS_MASTER_ENABLE | Master enable for outputs (0=force all outputs OFF, 1=allow normal control). |
| 00020 | 19 | R/W | Coil | NIGHT_MODE_ENABLE | Night mode enable (maps to existing device night mode concept if implemented). |

## 2.2 Discrete Inputs (1xxxx) — Inputs and link flags

Discrete Inputs are read-only status bits for inputs and system link flags.

| Ref | Offset | R/W | Type | Name | Description |
| --- | --- | --- | --- | --- | --- |
| 10001 | 0 | R | Discrete Input | DI1 | Digital input channel state (0/1). |
| 10002 | 1 | R | Discrete Input | DI2 | Digital input channel state (0/1). |
| 10003 | 2 | R | Discrete Input | DI3 | Digital input channel state (0/1). |
| 10004 | 3 | R | Discrete Input | DI4 | Digital input channel state (0/1). |
| 10005 | 4 | R | Discrete Input | DI5 | Digital input channel state (0/1). |
| 10006 | 5 | R | Discrete Input | DI6 | Digital input channel state (0/1). |
| 10007 | 6 | R | Discrete Input | DI7 | Digital input channel state (0/1). |
| 10008 | 7 | R | Discrete Input | DI8 | Digital input channel state (0/1). |
| 10009 | 8 | R | Discrete Input | DI9 | Digital input channel state (0/1). |
| 10010 | 9 | R | Discrete Input | DI10 | Digital input channel state (0/1). |
| 10011 | 10 | R | Discrete Input | DI11 | Digital input channel state (0/1). |
| 10012 | 11 | R | Discrete Input | DI12 | Digital input channel state (0/1). |
| 10013 | 12 | R | Discrete Input | DI13 | Digital input channel state (0/1). |
| 10014 | 13 | R | Discrete Input | DI14 | Digital input channel state (0/1). |
| 10015 | 14 | R | Discrete Input | DI15 | Digital input channel state (0/1). |
| 10016 | 15 | R | Discrete Input | DI16 | Digital input channel state (0/1). |
| 10017 | 16 | R | Discrete Input | DIN_DIRECT_1 | Direct GPIO input 1 (if used). |
| 10018 | 17 | R | Discrete Input | DIN_DIRECT_2 | Direct GPIO input 2 (if used). |
| 10019 | 18 | R | Discrete Input | DIN_DIRECT_3 | Direct GPIO input 3 (if used). |
| 10020 | 19 | R | Discrete Input | ETH_CONNECTED | Ethernet connected status (1=connected; link up and active). |
| 10021 | 20 | R | Discrete Input | WIFI_CONNECTED | WiFi connected status (1=connected). |
| 10022 | 21 | R | Discrete Input | AP_MODE_ACTIVE | Access Point mode active (1=AP mode). |
| 10023 | 22 | R | Discrete Input | RESTART_REQUIRED | Restart required flag (1=restart needed to apply some settings). |
| 10024 | 23 | R | Discrete Input | MODBUS_RTU_ACTIVE | Modbus RTU active (stack running and serial ready). |

## 2.3 Input Registers (3xxxx) — Live telemetry (fast polling)

Read-only telemetry intended for frequent polling by the VB.NET master.

| Ref | Offset | R/W | Type | Units | Name | Description |
| --- | --- | --- | --- | --- | --- | --- |
| 30001 | 0 | R | U16 | 0x0100 | MAP_VERSION | Register map version (0x0100 = v1.0). |
| 30002 | 1 | R | U16 | bitfield | DEVICE_STATUS | General device status bitfield (optional legacy). |
| 30003 | 2 | R | U16 | count | HEARTBEAT | Increments once per second (wrap allowed). Used for comm-health monitoring. |
| 30004..30005 | 3..4 | R | U32 | seconds | UPTIME_S | Uptime in seconds since boot (LOW at 30004, HIGH at 30005). |
| 30006 | 5 | R | U16 | count | CHANGE_SEQ | Increments whenever any DI/DO/AI/Sensor/RTC value changes. |
| 30007 | 6 | R | U16 | - | RESERVED_30007 | Reserved. |
| 30008 | 7 | R | U16 | - | RESERVED_30008 | Reserved. |
| 30009 | 8 | R | U16 | code | LAST_ERROR_CODE | Last error code (0 = OK). |
| 30010..30011 | 9..10 | R | U32 | bytes | FREE_HEAP_B | Free heap in bytes (LOW at 30010, HIGH at 30011). |
| 30012 | 11 | R | U16 | MHz | CPU_FREQ_MHZ | CPU frequency in MHz. |
| 30013 | 12 | R | U16 | 0..4095 | AI_RAW_1 | Analog input 1 raw ADC reading. |
| 30014 | 13 | R | U16 | 0..4095 | AI_RAW_2 | Analog input 2 raw ADC reading. |
| 30015 | 14 | R | U16 | 0..4095 | AI_RAW_3 | Analog input 3 raw ADC reading. |
| 30016 | 15 | R | U16 | 0..4095 | AI_RAW_4 | Analog input 4 raw ADC reading. |
| 30017 | 16 | R | U16 | mV | AI_MV_1 | Analog input 1 scaled millivolts (0..5000mV). |
| 30018 | 17 | R | U16 | mV | AI_MV_2 | Analog input 2 scaled millivolts (0..5000mV). |
| 30019 | 18 | R | U16 | mV | AI_MV_3 | Analog input 3 scaled millivolts (0..5000mV). |
| 30020 | 19 | R | U16 | mV | AI_MV_4 | Analog input 4 scaled millivolts (0..5000mV). |
| 30021 | 20 | R | S16 | 0.1C | DHT1_TEMP_X10 | DHT1 temperature (degC*10). |
| 30022 | 21 | R | U16 | 0.1%RH | DHT1_RH_X10 | DHT1 humidity (%RH*10). |
| 30023 | 22 | R | S16 | 0.1C | DHT2_TEMP_X10 | DHT2 temperature (degC*10). |
| 30024 | 23 | R | U16 | 0.1%RH | DHT2_RH_X10 | DHT2 humidity (%RH*10). |
| 30025 | 24 | R | S16 | 0.1C | DS18_TEMP_X10 | DS18B20 temperature (degC*10). |
| 30026 | 25 | R | U16 | bitfield | SENSOR_STATUS | bit0=DHT1 ok, bit1=DHT2 ok, bit2=DS18 ok. |
| 30027..30028 | 26..27 | R | U32 | unix s | RTC_UNIX_S | RTC UNIX time seconds (LOW at 30027, HIGH at 30028). |
| 30029 | 28 | R | U16 | year | RTC_YEAR | RTC year (e.g., 2026). |
| 30030 | 29 | R | U16 | 1..12 | RTC_MONTH | RTC month. |
| 30031 | 30 | R | U16 | 1..31 | RTC_DAY | RTC day. |
| 30032 | 31 | R | U16 | 0..23 | RTC_HOUR | RTC hour. |
| 30033 | 32 | R | U16 | 0..59 | RTC_MIN | RTC minute. |
| 30034 | 33 | R | U16 | 0..59 | RTC_SEC | RTC second. |
| 30035 | 34 | R | U16 | - | RESERVED_30035 | Reserved. |
| 30036 | 35 | R | U16 | - | RESERVED_30036 | Reserved. |
| 30037 | 36 | R | U16 | - | RESERVED_30037 | Reserved. |
| 30038 | 37 | R | U16 | - | RESERVED_30038 | Reserved. |
| 30039 | 38 | R | U16 | - | RESERVED_30039 | Reserved. |
| 30040 | 39 | R | U16 | bitmask | OUTPUTS_BITMASK | Outputs state bitmask (bit0=CH1 ... bit15=CH16). |
| 30041 | 40 | R | U16 | bitmask | INPUTS_BITMASK | Inputs state bitmask (bit0=IN1 ... bit15=IN16). |
| 30042 | 41 | R | U16 | bitmask | DIRECT_INPUTS_BITMASK | Direct inputs bitmask (bit0..2 used). |
| 30043 | 42 | R | U16 | flags | SYSTEM_STATUS_FLAGS | System status flags (bitfield; see section 3.1). |

# 3. Bitfields and bit-wise operations

## 3.1 IR30043 SYSTEM_STATUS_FLAGS

IR30043 is a U16 bitfield with the following bit assignments:

| Bit | Name | Meaning |
| --- | --- | --- |
| 0 | ETH_CONNECTED | 1 when Ethernet connected. |
| 1 | WIFI_CONNECTED | 1 when WiFi connected. |
| 2 | AP_MODE | 1 when AP mode active. |
| 3 | OUTPUTS_MASTER_ENABLED | 1 when outputs master enable coil is ON. |
| 4 | RESTART_REQUIRED | 1 when restart required to apply settings. |
| 5 | MODBUS_RUNNING | 1 when Modbus RTU stack is enabled and running. |
| 6..15 | RESERVED | Reserved for future use. |

Example: IR30043 = 0x0019 sets bits 0,3,4 (Ethernet connected + outputs enabled + restart required).

## 3.2 HR40006 CAPABILITIES bitmask

HR40006 is a U16 bitmask indicating supported features:

| Bit | Feature | Meaning |
| --- | --- | --- |
| 0 | Ethernet | Device includes Ethernet support. |
| 1 | WiFi | Device includes WiFi support. |
| 2 | WebServer | HTTP web server available. |
| 3 | WebSocket | WebSocket service available. |
| 4 | RS485 | RS485 interface available. |
| 5 | ModbusRTU | Modbus RTU available on RS485. |
| 6 | RTC | Real-time clock available. |
| 7 | Sensors | Sensors available. |
| 8 | Analog | Analog inputs available. |
| 9 | Schedules/Triggers | Schedules/triggers supported. |
| 10..15 | Reserved | Reserved for future features. |

# 4. Holding Registers (4xxxx) — Identity and configuration

## 4.1 Identity and strings (HR40001..HR40090)

String registers use ASCII encoding (2 chars per register).

| Ref | Offset | R/W | Type | Name | Description |
| --- | --- | --- | --- | --- | --- |
| 40001 | 0 | R | U16 | MAP_VERSION | Register map version (0x0100=v1.0). |
| 40002 | 1 | R | U16 | MODEL_CODE | Device model code (e.g., 0xA016). |
| 40003 | 2 | R | U16 | FW_VERSION_PACKED | Firmware version packed (MMmm). |
| 40004 | 3 | R | U16 | HW_VERSION_PACKED | Hardware version packed (MMmm). |
| 40005 | 4 | R | U16 | YEAR_OF_DEV | Year of development. |
| 40006 | 5 | R | U16 | CAPABILITIES | Capabilities bitmask (see section 3.2). |
| 40007..40010 | 6..9 | R | U16 | RESERVED | Reserved. |
| 40011..40026 | 10..25 | R/W | STRING(32) | BOARD_NAME | Board name (32 chars). |
| 40027..40042 | 26..41 | R/W | STRING(32) | SERIAL_NUMBER | Serial number (32 chars). |
| 40043..40058 | 42..57 | R | STRING(32) | MANUFACTURER | Manufacturer string (Microcode Engineering). |
| 40059..40066 | 58..65 | R | STRING(16) | FW_VERSION_STR | Firmware version string (optional). |
| 40067..40074 | 66..73 | R | STRING(16) | HW_VERSION_STR | Hardware version string (optional). |
| 40075..40090 | 74..89 | R/W | STRING(32) | DEVICE_NAME | Device name (matches web UI device name). |

## 4.2 MAC address configuration (HR40101..HR40124)

MAC bytes are stored as one byte per register (0..255).

| Ref | Offset | R/W | Type | Name | Description |
| --- | --- | --- | --- | --- | --- |
| 40101..40106 | 100..105 | R | U16 bytes | BOARD_EFUSE_MAC[6] | Base efuse MAC bytes (0..255). |
| 40107..40112 | 106..111 | R/W | U16 bytes | ETH_MAC[6] | Configured Ethernet MAC bytes; apply may require reboot. |
| 40113..40118 | 112..117 | R/W | U16 bytes | WIFI_STA_MAC[6] | Configured WiFi STA MAC bytes; apply may require reboot. |
| 40119..40124 | 118..123 | R/W | U16 bytes | WIFI_AP_MAC[6] | Configured WiFi AP MAC bytes; apply may require reboot. |

## 4.3 RS485 and Modbus settings (HR40201..HR40214)

Some settings may require reboot; firmware should set RESTART_REQUIRED accordingly.

| Ref | Offset | R/W | Type | Name | Description |
| --- | --- | --- | --- | --- | --- |
| 40201 | 200 | R/W | U16 | RS485_PROTOCOL | RS485 protocol (0=custom, 1=ModbusRTU). |
| 40202 | 201 | R/W | U16 | MB_SLAVE_ID | Modbus Slave ID (1..247). Default 1. |
| 40203 | 202 | R/W | U16 | MB_BAUD | Baud rate. Default 38400. |
| 40204 | 203 | R/W | U16 | MB_DATABITS | Data bits (7/8). Default 8. |
| 40205 | 204 | R/W | U16 | MB_PARITY | Parity (0=None, 1=Odd, 2=Even). Default 0. |
| 40206 | 205 | R/W | U16 | MB_STOPBITS | Stop bits (1/2). Default 1. |
| 40207 | 206 | R/W | U16 | FRAME_GAP_MS | Frame gap milliseconds (recommended 4..10). |
| 40208 | 207 | R/W | U16 | MB_TIMEOUT_MS | Device-side Modbus timeout (ms). |
| 40209 | 208 | W | U16 | APPLY_SAVE_SERIAL | Write 0xA55A to apply and save serial settings. |
| 40210 | 209 | R | U16 | APPLY_STATUS | Apply status (0=idle, 1=OK, 2=Error). |
| 40211 | 210 | R/W | U16 | USB_BAUD | USB serial baud rate (optional). |
| 40212 | 211 | R/W | U16 | USB_DATABITS | USB data bits (optional). |
| 40213 | 212 | R/W | U16 | USB_PARITY | USB parity (optional). |
| 40214 | 213 | R/W | U16 | USB_STOPBITS | USB stop bits (optional). |

## 4.4 Hardware configuration and analog calibration (HR40301..HR40326)

Analog calibration uses FLOAT32 (2 registers per value).

| Ref | Offset | R/W | Type | Name | Description |
| --- | --- | --- | --- | --- | --- |
| 40301 | 300 | R/W | U16 | OUTPUTS_BITMASK_WRITE | Desired outputs bitmask (bit0=CH1..bit15=CH16). |
| 40302 | 301 | R/W | U16 | OUTPUTS_WRITE_MASK | Write-mask: bit=1 applies corresponding bit from 40301. |
| 40303 | 302 | R | U16 | OUTPUTS_BITMASK_CURRENT | Current outputs bitmask (mirror of IR30040). |
| 40304 | 303 | R | U16 | INPUTS_BITMASK_CURRENT | Current inputs bitmask (mirror of IR30041). |
| 40311..40312 | 310..311 | R/W | FLOAT32 | AI_SCALE_CH1 | Analog scale factor channel 1 (FLOAT32; low word then high word). |
| 40313..40314 | 312..313 | R/W | FLOAT32 | AI_SCALE_CH2 | Analog scale factor channel 2 (FLOAT32; low word then high word). |
| 40315..40316 | 314..315 | R/W | FLOAT32 | AI_SCALE_CH3 | Analog scale factor channel 3 (FLOAT32; low word then high word). |
| 40317..40318 | 316..317 | R/W | FLOAT32 | AI_SCALE_CH4 | Analog scale factor channel 4 (FLOAT32; low word then high word). |
| 40319..40320 | 318..319 | R/W | FLOAT32 | AI_OFFSET_CH1 | Analog offset channel 1 (FLOAT32; low word then high word). |
| 40321..40322 | 320..321 | R/W | FLOAT32 | AI_OFFSET_CH2 | Analog offset channel 2 (FLOAT32; low word then high word). |
| 40323..40324 | 322..323 | R/W | FLOAT32 | AI_OFFSET_CH3 | Analog offset channel 3 (FLOAT32; low word then high word). |
| 40325..40326 | 324..325 | R/W | FLOAT32 | AI_OFFSET_CH4 | Analog offset channel 4 (FLOAT32; low word then high word). |

## 4.5 Safe maintenance commands (HR40610..HR40617)

Commands use a two-step arm/confirm sequence to prevent accidental resets.

| Ref | Offset | R/W | Type | Name | Description |
| --- | --- | --- | --- | --- | --- |
| 40610 | 609 | W | U16 | CMD_ARM | Write 0xA55A to arm command execution (valid for short window). |
| 40611 | 610 | W | U16 | CMD_CODE | Command code (see below). |
| 40612 | 611 | W | U16 | CMD_ARG0 | Optional argument 0. |
| 40613 | 612 | W | U16 | CMD_ARG1 | Optional argument 1. |
| 40614 | 613 | W | U16 | CMD_CONFIRM | Write 0x5AA5 to execute CMD_CODE if armed. |
| 40615 | 614 | R | U16 | CMD_STATUS | 0=idle, 1=armed, 2=running, 3=done_ok, 4=done_err. |
| 40616 | 615 | R | U16 | CMD_RESULT | Command result / details. |
| 40617 | 616 | R | U16 | CMD_LAST_ERROR | Last command error code (0=OK). |

## 4.5.1 Command codes (CMD_CODE)

Sequence: write HR40610=0xA55A (arm), write HR40611=code, then write HR40614=0x5AA5 (confirm).

| Code | Action |
| --- | --- |
| 1 | REBOOT |
| 2 | SAVE_CONFIG |
| 3 | RELOAD_CONFIG |
| 4 | FACTORY_DEFAULTS |
| 5 | CLEAR_LOGS |
| 6 | CONFIG_BACKUP_SAVE |
| 7 | CONFIG_BACKUP_RESTORE |

# 5. Recommended polling and control patterns

For a stable UI and low bus load:

Poll Input Registers 30001..30043 at a user-configurable interval (e.g., 250ms to 1000ms).

Use HEARTBEAT (30003) to detect connection loss; if it stops changing, reconnect.

Use CHANGE_SEQ (30006) to skip unnecessary UI updates if nothing changed.

Control DOs via coils 00001..00016, or bulk-write via HR40301/40302 if enabled in firmware.

Apply serial settings by writing 0xA55A to HR40209; if RESTART_REQUIRED becomes 1, execute REBOOT via safe commands.