CONTROLWORKS – BMS PROJECT ENGINEER | KEYWORD CHEAT SHEET

Speak from keywords → expand naturally (tech + process + outcomes).

| BMS + HVAC CONTROLS Chilled water: primary–secondary, ΔT, trim‑and‑respond, DP reset, staging, lead/lag, flow‑prove, anti‑short‑cycle PID: P then I, minimal hunting, live trending, fast loops (VAV) vs slow loops (hydronic) VAV unstable: sensor cal, actuator stroke, airflow/min‑flow, PID, trend (SP/PV/OUT), supply SAT, cross‑zone AHU/FCU issues: reversed actuator, sensor placement, safeties/interlocks, airflow cal, fan rotation, filters Validation: handheld calibration, wiring continuity, scaling/units, end‑switch feedback, fail‑safe return Energy: optimum start/stop, SAT reset, static‑P reset, DCV, night purge, CHW reset, equipment rotation, trending PROTOCOLS + INTEGRATION BACnet MS/TP: RS‑485, token‑passing, field devices, cost‑effective BACnet/IP: Ethernet, high bandwidth, supervisors/plant, scalable MS/TP trunk: MAC unique (0–127), baud consistent, 30–40 devices, polarity, termination, segmentation (routers) Modbus RTU: register map, data types, scaling, scanner test, point list, verify vs device display RS‑485 faults: address/baud/polarity, termination, isolate devices, analyser/capture TOOLS (SITE) Laptop (Niagara/BMS tools) • Multimeter • Manometer • Anemometer • Temp probe • RS‑485 analyser • Hand tools | NIAGARA N4 Stations: kitControl logic, PX graphics, alarms, histories, schedules, Haystack tagging Architecture: folders (equip/logic/points/alarms/hist/graphics), naming standards, templates (VAV/FCU) Discovery/mapping: validate network, import only required, rename/tag, check writable priorities Ops/security: backups (config.bog), restore, credentials, TLS/certs, version history, logs (Application Director) PROJECT DELIVERY Kickoff→handover: scope/drawings/SoO, clarify gaps, I/O schedule, logic+graphics, panel review, pre‑comm, site comm, tuning, demo/training, docs Variations: early identify, impact quantify, options, PM/client comms, document change control Prioritise: critical path, dependencies, access/safety, trade coordination, clear timelines COMMISSIONING FLOW AHU steps: wiring+sensors, mechanical readiness, safeties (fire/freezestat/DP), manual→auto, PID tune, alarms/trends/graphics Troubleshoot: trend evidence, isolate root cause, coordinate trade fix, retune, verify, document DOCUMENTATION + QUALITY Control sequences: mechanical intent → modes/safeties → step logic → review/validate I/O schedules: schematic ↔ panel ↔ controller mapping, loop checks, update as‑built QA: checklists, templates, consistent naming, revision control, clear handover packs BEHAVIOURAL KEYWORDS Calm under pressure • transparent updates • stakeholder comms • safety first Mentor/coach • standardise • ownership • client trust • engineering excellence |
| --- | --- |